# Detection Parameters – GUI Controls

This table summarizes the adjustable parameters added to the GUI, their role inside the processing pipeline, and the allowed ranges configured in the interface.

| Parameter | Meaning in the script | Allowed range |
| --- | --- | --- |
| Expected crystal diameter (µm) | Sets the crystal size scale for detection in PL images; used to compute min_sigma/max_sigma/num_sigma for blob_log via _sigma_range_from_expected_d_um. | 0.5 – 50.0 µm |
| Crystal detection sensitivity | Controls how easily a blob is considered a crystal; maps to blob_log(..., threshold=...). Lower values detect more (risk FPs), higher values detect fewer (risk FNs). | 0.005 – 0.08 |
| Cell split aggressiveness (µm) | Controls separation of touching algae cells; sets peak_local_max(min_distance=...) (converted from µm to pixels) for watershed seeding. | 0.5 – 20.0 µm |
| Cell mask threshold bias | Bias factor applied to the BF threshold. Multiplies the chosen threshold (Otsu/Li/IsoData) in _auto_threshold_with_bias; <1.0 more permissive, >1.0 more conservative. | 0.80 – 1.20 |